# Week 10 – TCP/UDP

Class:P-CB-06

Student numbers: 4252861

Student names:  Tobias Halomoan

Date: Aug 2019
Version 1.0

# TCP/UDP

Task 1: TCP in Netcat

To do this assignment we will use the Netcat tool which is provided in the Netkit. Netcat makes it possible to create and use TCP/UDP connections. If you want more info about Netcat you can consult Internet. To make this assignment we will reuse the net_routing lab from the previous assignments. Let’s start a chat session by connecting 2 netcat instances via a TCP connection.

To listen to the TCP connections, go to one of your simulated nodes (e.g. PC1A) and issue the following command:

nc –l –p <port_nr>

This will make netcat listen to port number that you have specified in port_nr and accept connections.

Note: Any port number would be ok, as long as it is not used by another application.

To establish a TCP connection you can issue the following command from another simulated node (e.g. PC1C)

- nc <IP address of the “listening” node> <port_nr of the “listening node”>

This will make a TCP connection with the listening netcat instance.Now you can chat from one netcat instance to the another. Try it out!

Your task:

- Netcat can also be used to copy the contents of a file from one place (file, folder, computer) to another. Find out how and try it out.

Provide screenshots of the sending and receiving command.

Let’s now build a basic one-page webserver using netcat.

As a first step create a textfile ‘response.txt’ with following content:

HTTP/1.1 200 OK

Content-Type: text/html;

Content-Length: 12

Connection: close

Hello World!

This is a proper HTTP response.

You are going to simulate HTTP server and HTTP client (browser) using netcat. You are going to use Gateway node for running HTTP client (browser), see section below about how to install links browser on Gateway before you start the exercise. You are going to use any node of your network (e.g. PC2C) to simulate HTTP server.

Your task:

Construct an appropriate netcat command to listen to a port on your HTTP server (e.g. PC2C) and send the contents of the file response.txt to the HTTP client (Gateway) when a connection is made to this port. That is roughly what a webserver does too.
You can test it by entering the following URL in links browser on Gateway.

http://<IP_ADDRESS_OF_WEBSERVER>:<port_nr> 
If everything works well the links should show the “Hello World” webpage.

Links installation:

Before starting this task, you have to install links text-mode web browser. Follow these steps:

- Go to your lab’s Gateway directory. Create etc subdirectory and create there a resolv.conf text file with the following contents:

nameserver 8.8.8.8
search localdomain

- Start your lab. Now you should be able to connect to the Internet, so do the following installation on the Gateway node:

apt-get install links

Now the links web browser should be available on your Gateway node. Watch out, once you stop your Gateway node, you have to do ‘apt-get install links’ command again to install links.

Note1: You can start links browser by issuing this command:

links

To be able to enter the URL in links browser press “G”.

Note2 : You can put the response.txt file in your net_routing directory  before start of the lab (lstart command). After starting the lab, you can find this file in the /hostlab directory of your node.

Provide a screenshot of the netcat command you used and of the links browser output.

Task 2: Find 2 TCP uses

Think about two different scenarios for TCP use that you can simulate (you can do this on your own PC, so you don’t need Ubuntu for this). Start a Wireshark trace for both scenarios.

Describe the chosen scenarios and a proof of TCP use in them by attaching a Wireshark trace showing TCP packets.

Scenario 1 : HTTPS Traffic

We capture HTTPS Traffic through opening multiple websites for a couple of seconds. We found traffic to the port 443.  HTTPS is used for secure communication over computer networks and is widely used in the internet.

Scenario 2: SMTP Mail Server

We connect to the Google SMTP Mail Server using the Windows Telnet client to generate the traffic. The SMTP is a widely used communications protocol for email transmission. The TCP port was port 587.

Choose one of the 2 scenarios traces and browse it in the Wireshark. Select ‘Statistics > Flow Graph’ and then choose flowtype ‘TCP flow’ to draw a Sequence Diagram of the TCP message interaction that you see in Wireshark.

SMTP Sequence Diagram:

The Beginning:

The client (my computer) establishes a connection to the server, using the SYN (Synchronize sequence number) which informs server that client is ready to communicate.

Then, the server responds with SYN + ACK signal, ACK (Acknowledgement) signifies the OK signal from the server side, and SYN signifies which sequence number it will try to start the transfer with.

The final ACK from the client side signifies the OK signal from the client side, finishing the “Three Way Handshake” and establishing a reliable connection between the two.

The LEN values signifies the length of the TCP payload in bytes. The SEQ value signifies the sequence number for communication, and the ACK value indicates that the server has acknowledged the client’s SYN flag and will proceed with the transfer.

The Middle:

In the middle, data transfer is done by continuous transfers and acknowledgement between the two parties. The PSH is the PUSH flag, that signifies the sending of data even when the buffer is not full during transfers!

In the End:

In the end, the FIN (FINISH) flag is established. This flag signifies the end of data transfer and the closing of the connection.

Task 3: Find 2 UDP uses

Think about two different scenarios for UDP use that you can simulate (you can do this on your own PC, so you don’t need Ubuntu for this). Start a Wireshark trace for both scenarios.

Describe the chosen scenarios and a proof of UDP use in them by attaching a Wireshark trace showing UDP packets.

Scenario 1: Renew DHCP:

Using the ipconfig /renew command in the terminal, my DHCP assigned IP address is renewed. This will generate UDP traffic, particularly in port 68. Dynamic Host Configuration Protocol is a network management protocol that dynamically assigns an IP address to devices.

Scenario 2: Reverse DNS Lookup (rDNS)

By using the command nslookup 8.8.8.8, the Domain Name System protocol, which uses UDP, looks up the aforementioned IP address. DNS is a naming system that acts as a “finder” which translates domain names to the numerical IP addresses needed for locating networks under many different networking protocols. Reverse DNS Lookup generates traffic particularly in port 53.